Numbering Systems

Points to cover:

1.   Decimal Numbering System

·    Base 10: explain why it is a base 10 system; there are numbers from 0 -9

·    Explain what are weights and give an example

| Weight | Value |
| --- | --- |
| 100 | 1 |
| 101 | 10 |
| 102 | 100 |
| 103 | 1000 |
| 104 | 10000 |

102

101 100

567

5 x 100 + 6 x 10 + 7 x 1 è 500 + 60 + 7 è 567

Section Questions

Please answer these questions with them so they can answer the sheet

1.   What is the weight of digit 6 in 1386?

Answer: 100 = 1

2.   Express 100 as a power of ten

Answer: 100 = 102

2.   Binary Numbering System

·    Base 2: Explain why it is a base 2 system; there are number s  from 0 and 1

·    Explain the weights in a binary system and give an example

2-Binary Weights

| Weight | Value |
| --- | --- |
| 20 | 1 |
| 21 | 2 |
| 22 | 4 |
| 23 | 8 |
| 24 | 16 |
| 25 | 32 |
| 26 | 64 |
| 27 | 128 |

22      21     20

101

1 x 4 + 0 x 2 + 1 x 1 è 4 + 0 + 1 è 5

Section Questions

Please answer these questions with them so they can answer the sheet

3.   What is the highest decimal numbe r that can be represented by 5 numbers of binary digits (bits)?

Answer: 5 bits è maximum number all 1s è 11111 in binary è convert to decimal è

31 in decimal(1+2+4+8+16)

3.   Binary to Decimal Conversion

·    Explain how to conve rt from binary to decimal using the sum-of-weights method

·    Give an example

23   22    21     20

1010

1 x 8 + 0 x 4 + 1 x 2 + 0 x 1 è 8+ 0 + 2 + 0 è 10

4.   Decimal to Binary Conversion

·    Explain how to conve rt from decimal to binary using the sum-of-weights method

·    Give an example

48

i.   What’s the largest binary numbe r that can be used to represent 48?

25è32

| 25=32 | 24=16 | 23=8 | 22=4 | 21=2 | 20=1 |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |

ii.   48 – 32 = 16

iii.   What’s the largest number that can represent 16? 24è 16

| 25=32 | 24=16 | 23=8 | 22=4 | 21=2 | 20=1 |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 |  |  |  |  |

iv.   16 - 16=0

| 25=32 | 24=16 | 23=8 | 22=4 | 21=2 | 20=1 |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 0 | 0 | 0 |

·    Explain how to conve rt from decimal to binary using the divide-by-2 method

| Number | Divide by 2 | Carry (0 = no carry/1=carry) |
| --- | --- | --- |
| 48 | 24 | 0 |
| 24 | 12 | 0 |
| 12 | 6 | 0 |
| 6 | 3 | 0 |
| 3 | 1.5 | 1 |
| 1 | 0.5 | 1 |

·    Give an example

48

Write this way

up

110000

Section Questions

Please answer these questions with them so they can answer the sheet

5. How many bits are required to represent 17?

Answer: Convert 17 to binary è 10001 ècount the bits è 5 bits

5.   Hexadecimal Numbers:

- Hexa means 16. In the hexadecimal number system, it uses 16 digits. It consists of numbers and alphabets. It includes numbers 0, 1, 2, 3, 4, 5, 6, 7, 8, 9 and A, B, C, D, E, F; total 16 digits.

- It is denoted by s16, where s is a hexadecimal number.

- Examples:    1) A116       2) EE916      3) FD65416

Question 1: Convert A2B16 to an equivalent binary number.

Solution: Given hexadecimal number = A2B16

First, convert the given hexadecimal to the equivalent decimal number.

A2B16 = (A × 162) + (2 × 161) + (B × 160)

= (A × 256) + (2 × 16) + (B × 1)

= (10 ×256) + 32 + 11

= 2560 + 43

= 2603(Decimal number)

Now we have to convert 260310 to binary

The binary number obtained is 1010001010112

Hence, A2B16 = 1010001010112

Question 2: Convert E16 to an equivalent binary number.

Solution: Given, a hexadecimal number is E.

First, convert the given hexadecimal to the equivalent decimal number.

E16 = E × 160

= E × 1

= E

=14 (Decimal number)

Now we have to convert 1410 to binary number.

The binary number obtained is 11102

Hence, E16 = 11102